### ANALOG COMMUNICATION TRAINERS

### Amplitude Modulation & Demodulation Trainer

  It has got three parts namely carrier generator, AM modulator And AM demodulator.

  Built in fixed power supplies of +12V,-12V. @ 250mA

  Carrier generator using 8038 IC with fixed frequency 100 KHz and fixed amplitude.

### Frequency Modulation & Demodulation Trainer

  Built in fixed power supplies of +12V,-12V @ 250mA

  Carrier generator using 8038 IC with fixed frequency 50KHz and fixed amplitude

  Direct method is used for modulation circuit with RC combination

  Phase lock loop 565IC and Op-Amp 741 IC is used in demodulator circuit.

### Balanced Modulator Trainer

  Built in a fixed power supplies of +12V, -12V @ 250mA

  1496 IC is used as balanced modulator.

  Two potentiometers are provided to vary the carrier suppression

### Characteristics of Mixer Trainer

  Built in fixed power supply of +12V @ 250mA

  2N 2369 NPN transistors is used as frequency mixer.

### Synchronous Detector Trainer

  Built in fixed power supplies of +12V, -12V. 250mA

  Built in variable carrier generator from 50KHz to 150KHz.

  Built in AM modulator circuit with external AF frequency and 1496 
    IC is used as synchronous detector.

### Pre-Emphasis & De-Emphasis Trainer

  Built in fixed power supply of +12V @ 250mA

  Two different inductors are used with NPN transistor in Pre – emphasis circuit.

  Two different resistors and capacitors are used in demodulator circuit.

### SSB System Trainer

  Built in fixed power supplies of +12V,-12V, +5V, -5V

  RF generator with fixed frequency 100 KHz & variable amplitude 0-1.5V

  AF generator with variable frequency 0-5KHz

  Variable amplitude 0-10V. Automatic gain control (AGC)

  Variation to adjust wave shape.

  Two balanced modulators.

  LSB, USB, demodulation blocks individually on one kit Phase shifter method used for modulation

### Receiver Measurement Trainer

  Built in fixed power supplies of + 15V, -15.

  AF generator with variable frequency 200Hz to 10KHz 
    and with variable amplitude.

  RF generator with variable frequency 300KHz to 1200KHz
    and with variable amplitude.

  Digital display for both RF & AF frequency readout.

  IC 1619 is used for AM receiver measurements.

  Built in AM modulator.

### Phase Lock Loop Trainer (using 565) Trainer

  Built in fixed power supplies of +5V,-5V

  565 IC , 741 IC and 7490 IC divided by 10 counter is given on board .

  Different values of resistors and capacitors

  One 10Kohms Potentiometer is given to vary the output frequencies.

### Phase Modulation Trainer

  Built in fixed power supplies of +15V, -15V

  IC 2205 is used as phase modulator.

### Frequency Synthesizer Trainer

  Built in fixed power supplies of +5V, -5V

  Built in fixed frequency of 1KHz. 565 IC , Divided by counter provided on board.

  Different values of resistors and capacitors provided on board.

### AGC Characteristics Trainer

  Built in fixed power supplies of +15V, -15V, +5V.

  AF generator with variable frequency 200Hz to 10KHz and with variable amplitude.

  RF generator with variable frequency up to 1200KHz and with variable amplitude.

  IC 1619 is used for AM receiver measurements.

  Built in AM Modulator