### DIGITAL COMMUNICATION TRAINERS

### Pulse Amplitude Modulation and Demodulation Trainer

  Built in fixed power supplies of +12V

  555 IC –timer as a RF generator with 400 KHz fixed frequency

  The modulation is controlled with a CMOS sampling switch 4016 IC

  BC 107 is used for demodulation in CE configuration

### Pulse width modulation and Demodulation Trainer

  Built in fixed power supply of +12V -12V

  555 IC is used as clock generator with fixed frequency of 20KHz and fixed amplitude.

  LM 324 IC is used as AF generator with fixed frequency of 500Hz and Variable amplitude

  555 IC is used as modulator and Op-Amp 324 IC is used a demodulator.

### Pulse Position Modulation & Demodulation Trainer

  Built in fixed power supplies of +12V,-12V

  555 IC is used as clk generator with fixed frequency of 20KHz & fixed amplitude.

  LM 324 IC is used as AF generator with fixed freq. of 500Hz and Variable amplitude

  555 IC is used as modulator and Op-Amp 324 IC is used a demodulator

### Frequency Shift Keying Modulation & Demodulation Trainer

  Built in fixed power supplies of +12V,-12V, +5V,-5V

  4051 IC for modulator circuit.8038 IC for carrier generator .

  7490 IC for data generator 324 IC for Demodulator

### Pulse Code Modulation & Demodulation Trainer

  Built in fixed power supplies of +12V,-12V ,+ 5V

  Variable AF generator 20Hz to 150Hz

  Clock generator with fixed frequency of 10KHz

  Data output with LED variations Variable DC voltage – 5V to +5V

  ADC 800 IC, 74 LS 165 IC are used in modulation

  74 LS 164 IC, 74 LS 374 IC (buffer) , DAC 800IC & LPF are used in Demodulation

### Differential Phase Shift Keying Modulation and Demodulation Trainer

  Built in fixed power supplies of +12V, -12V , +5V

  8038 IC is used as data generator

  7490 IC is used as data generator .4051 IC, 7474 IC, 741 IC 
    are used in modulator 7486 IC is used in demodulator circuit

### Delta Modulation & Demodulation Trainer

  Built in fixed power supplies of +12V,-12V , +5V

  555 IC is used as clock generator TL084 IC is used as AF generator

### Phase Shift Keying Modulation & Demodulation Trainer

  Built in fixed power supplies of +12V,-12V, +5V

  8038 IC is used as carrier generator and 7490 IC is used as data generator

  4051 IC, 324 IC are used in modulator circuit

  7486 IC is used in demodulator circuit

### Differential Pulse Code Modulation & Demodulation Trainer

  Variable AF generator 15Hz to 30Hz.

  Variable DC voltage -5V to +5V.

### Ask Modulation & Demodulation Trainer

  Built in fixed power supplies of +12v,-12v, +5v, -5v.

  8038 IC is used as carrier generator and 7490 IC is used as data generator.

  4051 IC, 324 IC are used in modulator circuit.

  7486 IC is used in demodulator circuit

### Time Division Multiplexing & De-multiplexing Trainer

  In built carrier generator

  3 different frequency data signals are generated by using 74163 IC

  Built in fixed power supply +5V, -5V .Modulation & demodulation 
    are individually provided on board by using 4051 IC

### Sampling Theorem Verification Trainer

  Built in fixed power supplies of +12V,-12V

  Variable clock frequency 3-30KHz (low) and 6-60 KHz (High)

  4016 IC is used for sampling and LM 324 IC is used for reconstructing the signal

  Additional Op-Amp and CMOS are provided for external

### QPSK modulation and demodulation Trainer

  In built fixed power supply of +5V @ 250mA.

  In built carrier generator and data generator.

  To change data signal a switch is provided on board

  Modulator, phase detector, bit pair detector, demodulator are provided on board.

### Companding Trainer

### Frequency Division Multiplexing & Demultiplexing Trainer